# METHOD OF STATEMENT

Date: 16 June 2014

Description of work

To hoist down 2 unit machine from 2nd & 3rd floor by using platform and mobile/lorry crane.

Load onto lorry chassis and deliver to new location

Machine Spec

1 unit Mori Seiki NL 2500 CNC Lathe (Wt: 5 ton)

1 unit Mori Seiki NH 4000 Horizontal Milling (Wt: 9 ton)

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

To be advised

Location of Operation

#### No 6 Woodlands Loop

(MMI)

- Equipment Use

1 x 50 ton mobile crane

1 x 50 ton mobile/lorry crane

1 x platform

Rollers

Wire/webbing sling

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Lifting crew is pre-briefed prior to commencement of job                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc) Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Barricade operation area with safety cones, red/white tape

- Hoist up platform to 2nd level opening

- Tagline will be use to secure and control the swing motion of platform

- Stabilize platform and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton mobile/lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton crane to hoist down machine from 2nd floor

- Load machine onto lorry, secured and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175