# METHOD OF STATEMENT

Date: 01 Aug 2014

Description of work

To hoist down 2 unit machine from 3rd floor by using platform and mobile crane.

Load machine onto lorry and deliver to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

01/08/2014

Location of Operation

#### No 88 International Rd

Hunting Energy Services Pte Ltd

- Equipment Use

1 x 100 ton mobile crane

Lorry crane

1 x platform

Wire sling

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will clear and dispose

Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Lifting crew is pre-briefed prior to commencement of job                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc) Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Barricade operation area with safety cones, red/white tape

- Hoist up platform to 3rd level opening

- Use tagline to control the swing motion of platform

- Stabilize platform and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out and load machine onto platform

- Tied and secured machine onto platform with strapping belt

- Hoist down platform together with machine

- Load machine onto lorry, secured and deliver to new location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175