# METHOD OF STATEMENT

Date: 13 Feb 2014

Description of work

To unload machine and accessories from container, unpack, shift and position into customer premise

Date / Time of Operation

19/02/2014   -   09:30am

Location of Operation

#### No 61 Seletar Aerospace View

- Equipment Use

1 X 50 ton mobile crane

Lorry crane

Heavy duty rollers / pallet truck

Wire sling / shackle

1 x Forklift

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather.

All debris will clear and dispose

- Lifting Crews

MOM certified crane operator, lifting supervisor, rigger man and signalman will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel is to be pre-briefed before commencement of work

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc.)                                                                                                                                               Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger man, signalman

# JOB SCOPE

- Setting up of 50 ton mobile crane at designated lifting area

- To barricade operation area with safety cones, red/white tape and “lifting in operation” signboard

- Hoist up machine from FR container and load onto the ground

- Uncrate machine case

- After uncrate, lifting supervisor will instruct rigger man to hook wire sling at correct lifting point of machine

- When all sling have been secured, signalman will give instruction to crane operator to lift up machine to a height of about 4” to 6” from machine base

- If stability and balancing is not achieved, lifting supervisor will stop hoisting and rigger man will re-adjust the sling or adding shackle till the stability is achieved

- Hoist machine away from base and slowly swing to loading area

- Lower machine down slowly

- Workers to insert rollers below machine for the move

- Shift and position machine to the designated area

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Regards: Zailani

Tel:62824175