# METHOD OF STATEMENT

Date: 24th Sep 2020

Description of work

To shift out 1 unit machine and accessories from production to loading bay, load onto lorry chassis and deliver to new factory

Risk Assessment

A copy of Risk Assessment will be attach.

Date of Operation

29/09/2020 (Tuesday)

Time of Operation

Afternoon schedule

Location of Operation

#### Micro-Mechanics Pte Ltd

Move out: Blk 2 Kaki Bukit Ave 1 #01-01

Move in: Blk 4 Kaki Bukit Ave 1 #01-07

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Deploy lorry crane at designated loading/unloading bay

- Shift out machine from customer premise at ground floor to loading bay

- Load machine onto lorry chassis

- Secured machine with strapping belt

- Transport machine to new factory

- Reverse lorry loader to loading bay

- Flush up lorry to level with loading bay

- Shift out machine from lorry chassis

- Transport machine into tenants premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175