# METHOD OF STATEMENT

Date: 12th Jan 2023

Description of work

To shift out 1 unit machine to loading area, hoist up and load onto lorry chassis, transport to another location

Machine Spec

1 unit Tigo 565 CMM

Dim: 170 x 155 x 255 cm

Wt: 1, 100 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

15/02/23

Time of Operation

09:45 am

Location of Operation

#### 6 Fusionopolis Way #01-06,

#### SIMTech Synthesis Building

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Plastic sheet (to cover up machine)

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

Any debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Shift out machine to loading area

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and load machine onto lorry chassis

- Release all sling from crane hook

- Secured machine with strapping belt

- Transport to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175