# METHOD OF STATEMENT

Date: 12th Feb 2025

Description of Work

To dismantle machines into parts using hydraulic gantry crane, shift out to outside factory for packing, after packing load up onto customer arranged container and secured

Machine Spec

Refer to quotation

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate from:

19/02/25 - set up gantry crane

20/02/25 – dismantle machine parts

25/02/25 – packing of machine

Time of Operation

09:45 am

Location of Operation

51 Corporation Rd

Avitron Pte Ltd

Equipment Use

Lorry loader

Hydraulic gantry crane

1 x 4.5ton forklift

Heavy duty rollers

Wire sling

Webbing sling

Shackle

Obstacles

No obstacles on operation area and route to machine position

Barricade off operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle or personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- To unload gantry crane set from low loader using lorry crane

- Shift gantry crane into factory premise

- To assembly gantry crane inside factory

- Assist customer engineer to dismantle machine parts

- Rigger will attach required lifting gears onto gantry crane which is to be secure to the load to be lifted

- Once rigging is complete, gantry crane to lift up machine by parts

- Shift out gantry crane

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift out to loading area

- Rigger will attach required lifting gears to crane hook which is to be secure to the load to be lifted

- Lorry crane to hoist up machine and load onto wooden base for packing

- After packing, load up onto customer arranged low loader/lorry/container

- Lashing to secured the load

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175