# METHOD OF STATEMENT

Date: 27th Jan 2023

Description of Work

To dismantle 7 unit machine into parts using hydraulic gantry crane, shift out to outside factory for packing, after packing load up onto customer arranged lorry/container and secured

Machine Spec

Refer to quotation

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03/04/23   -   31/08/23

Time of Operation

09:45 am

Location of Operation

51 Corporation Rd

Avitron Pte Ltd

Equipment Use

70-100 ton mobile crane

Lorry loader

2 set hydraulic gantry crane

1 x 4.5ton forklift

Heavy duty rollers

Wire sling

Webbing sling

Shackle

Obstacles

No obstacles on operation area and route to machine position

Barricade off operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle or personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- To unload 2 set of gantry crane from low loader using lorry crane

- Shift gantry crane into factory premise

- To assembly gantry crane inside factory

- Lifting personnel will attached all required lifting gears onto gantry crane which is to be secure to the load to be lifted

- Once rigging is complete, gantry crane to lift up machine by parts

- Shift out gantry crane

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift out to loading area

- Mobile crane to hoist up machine and load onto wooden base for packing

- After packing, load up onto customer arranged lorry/container

- Lashing to secured the load

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175