# METHOD OF STATEMENT

Date: 10th May 2022

Description of Work

To shift out 1 unit press machine to outside factory, using hydraulic gantry crane to lift up machine and load onto cometto trailer, transport and deliver to new factory

Machine Spec

1unit SF450 Press

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

17/05/22 – 19/05/22

Time of Operation

09:45 am

Location of Operation

Shimano (S) Pte Ltd

20 Benoi Sector – 11 Bulim Walk

Equipment Use

2 set hydraulic gantry crane

Low-loader

Cometto trailer

1 x 4.5ton forklift

Wire/webbing sling

4 pcs 55ton shackle

4 pcs 25ton shackle

Obstacles

No obstacles on operation area and route to machine position

Barricade off operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle or personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- To unload and set up 2 set of gantry at outside factory premise

- Shift out machine from existing location to near gantry crane

- Lifting personnel will attached all required lifting gears onto gantry crane which is to be secure to the load to be lifted

- Once rigging is complete, gantry crane to lift up machine to required height

- Reverse cometto trailer to below machine

- Lower down machine onto cometto

- Release all sling from machine hoisting point

- Secure machine onto cometto with chain lock

- Transport and deliver machine to new factory

- Unload and position machine at designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175