# METHOD OF STATEMENT

# Reference No: EW/Q/0425/24

Date: 06th May 2024

Description of work

1) To shift out and hoist down 2 unit Milling and 1 unit Lathe machine from 2nd floor to the ground, shift and position into workshop

2) To hoist down 1 unit Milling machine and 2 unit Lathe machine from 2nd floor, load up onto lorry chassis and deliver to another building

Machine Spec

To be hoist down

From ARC LHN-B3-02 – MP1, LHN-B4-03

2 unit Milling machine / Wt: 2, 000 kg

1 unit Lathe machine / Wt: 1, 000 kg

By using lift

T-Jaw machine

Heavy duty rack

Bench Drill

Famepla 40” tool cabinet

Famepla 47” tool cabinet

2 x pallet

From ARC LHN-B3-02 – Innovation @ MAE Lab N3-B4a-02a

To be hoist down

1 unit Milling machine / Wt: 2, 000 kg

1 unit Lathe machine / Wt: 1, 000 kg

From ARC LHN-B302 – Modal Shop N3-B4C-03

To be hoist down

1 unit Lathe machine / Wt: 1, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/05/24 (Tuesday)

Time of Operation

09:45 am

Location of Operation

#### 63 Nanyang Drive, LHN-B3-02

The ARC – Leaning Hub North

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

Webbing sling

Shackle

Plywood (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the hoisting work

- Setting up of crane on site

- Crane operator to extract the crane boom to 2nd floor

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine

- Hoist up 1ft above ground to check on balancing

- When well balance, hoist down machine to the ground

- Load up machine onto lorry chassis

- Release all sling from machine rigging point

- Secured machine with strapping belt

- Transport machine to another building

- Unload, shift and position machine into workshop at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175