# METHOD OF STATEMENT

Date: 15th June 2023

Description of work

To shift out and hoist down 2 unit equipment from 2nd floor to the ground and load up onto lorry chassis

Equipment Spec

To be hoist down

1 unit Pressure Tank

Dim: 140 x 140 x 140 cm / Wt: 400 kg

By using lift

1 unit 350V 3D Printer

Dim: 132 x 100 x 120 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Time of Operation

09:45 am

Location of Operation

#### 65 Nanyang Drive

LHN-B3-04

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the hoisting work

- Set up crane on site

- Crane operator to extract the crane boom to 2nd floor

- Insert sling to crane hook and secure onto rigging point of equipment

- Hoist up 1ft above ground to check on balancing

- When well balance, hoist down equipment to the ground

- Load up equipment onto lorry chassis

- Release all sling from equipment rigging point

- Secured equipment with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175