# METHOD OF STATEMENT

Date: 31st Oct 2022

Description of Work

To unload 8 packages of 1 set machine from 1 x 40ft FR and 3 x 40ft HQ container using hydraulic gantry crane, uncrate, shift and position into factory premise using rollers

Machine Spec

1 unit DMU 210P machine

Refer to packing list attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

05/11/22 – 11/11/22

Time of Operation

09:30 am

Location of Operation

16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

2 set hydraulic gantry crane

633 lorry crane

Low-loader

1 x forklift

1 x boom/scissor lift

Wire sling

Shackle

Hydraulic jack

Electrical/air rollers

Hard wooden block

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacles on operation area and route to machine position

Barricade off operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- To unload 2 set of gantry crane from low loader using lorry crane

- To assembly gantry crane outside factory premise

- Reverse FR container under gantry crane

- Lifting personnel will attached all required lifting gears onto gantry crane which is to be secure to the load to be lifted

- Once rigging is complete, gantry crane to lift up case from FR container

- Move out container

- Lower down case to the ground

- Uncrate case and put aside

- Lifting personnel will attached all required lifting gears onto gantry and secure to rigging point of machine

- Lift up and detach machine from base

- Lower down machine to near ground and insert rollers under machine bed

- Transport machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175