# METHOD OF STATEMENT

Date: 18th Nov 2025

Description of Work

To dismantle machine into parts using gantry crane, shift out to outside factory, using hydraulic gantry crane to load up onto lowbed & cometto

Machine Spec

1 x Patec 600ton Press – to be dismantle to 3 parts

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

25/11/25 – 09/12/25

Time of Operation

09:30 am

Location of Operation

70 Joo Koon Circle

Nakano Metal Press Pte Ltd

Equipment Use

Lorry loader

2 set hydraulic gantry crane

1 x 4.5ton forklift

Heavy duty low-bed

Cometto

1 x boom lift

Heavy duty rollers

Wire/webbing sling

Shackle

Obstacles

No obstacles on operation area and route to machine position

Barricade off operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle or personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- To unload 2 set of gantry crane from low loader using lorry crane

- To assembly gantry crane inside factory premise

- Use of boom lift to assist when working at height

- Lifting personnel will attach required lifting gears onto gantry crane and secure to rigging point of machine

- Once rigging is complete, gantry crane to lift machine by parts

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift out machine to loading area

- Dismantle gantry crane

- Shift gantry to outside factory

- Gantry crane to lift up machine

- Reverse lowbed to under machine

- Lower down and load machine onto lowbed chassis

- Secured machine with chain belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175