# METHOD OF STATEMENT

Date: 15th Oct 2019

Description of work

To unload 2 cases of 1 unit machine from 1 x 40ft FR and 1 x 40ft HC container using forklift, uncrate, shift and position into customer premise at ground floor

Machine Spec

MEGA 130 X 7000

Case 1: 11500 x 3300 x 3300 mm

G/Wt: 23, 500 kg

Case 2: 11100 x 2280 x 2350 mm

G/Wt: 1, 400 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 25/10/2019

Time of Operation

09:45 am

Location of Operation

#### No 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 30 ton forklift

1 x 3 ton battery forklift

1 set rollers

1 set hydraulic jack

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane and forklift operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety glass)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- 30 ton forklift to fork down case from container

- Unload to the ground

- Uncrate case

- Fork up and detach machine from base

- Transport machine to door entrance

- Lower down machine to near ground level

- Insert rollers to machine base

- Shift and transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175