# METHOD OF STATEMENT

Date: 15th Aug 2024

Description of work

To shift out 5 unit machine and accessories to loading area, using forklift to fork up and load onto lorry chassis and deliver to new factory

Machine Spec

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/08/24 (Thursday)

Time of Operation

10:00 am

Location of Operation

#### Blk 1 A.M.K Pk 2A #01-07

Hiang Li Engineering

Equipment Use

2 x lorry crane

1 x lowbed (to transport forklift)

1 x 10 ton forklift

Rollers/pallet truck

Hand jack/woodblock

Obstacles

No obstacle on unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Using 10ton forklift to fork down machine from lorry chassis

- Transport machine to unloading area

- Lower down machine onto woodblock

- Insert rollers under machine bed

- Shift and transport into tenant premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175