# METHOD OF STATEMENT

Date: 5th Dec 2018

Description of work

To unload 4 packages of 1 unit machine and accessories from lorry chassis using lorry crane, insert pallet truck/rollers to machine base, shift and position into customer premise at ground floor

Machine Spec

1 unit Akira Seiki RMV 250T

Machine Dim: 199 x 227 x 247 cm

Wt: 3, 865 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

10/12/2018   -   09:30 am

Location of Operation

#### 10 Loyang Lane

Pratt & Whitney

Equipment Use

2 x lorry crane

Pallet truck/rollers

Wire/webbing sling

Metal sheet/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated unloading area

- Hoist up machine from lorry chassis and lower down to near ground level

- Swing crane slowly to loading area

- Insert pallet truck/rollers to machine base

- Shift and transport machine into premise

- Position machine at the designated location

- Unload machine from pallet truck/rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175