# METHOD OF STATEMENT

Date: 14th May 2019

Description of work

To shift out 1 unit machine to loading bay, using lorry crane to hoist up machine, load onto lorry chassis and transport out of factory premise

Machine Spec

Dim: 270 x 250 x 240 cm

Wt: 2, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/05/2019

Time of Operation

09:45 am

Location of Operation

#### 11 Tuas South Ave 12

Halliburton Completions MFG Pte Ltd

Equipment Use

1 x lorry loader – PALFINGER PK50002EH

Pallet truck/rollers

1 set wire/webbing sling

1 set 8.5 ton shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Shift out machine from production to loading bay

- Using lorry crane to hoist up machine

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to being transport out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175