# METHOD OF STATEMENT

Date: 27th June 2025

Description of work

To shift out machineries from production onto platform, using lorry and forklift to load up onto customer arrange container

Machine Spec

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate: 3 days job

11/07/25  -  13/07/25

Time of Operation

10:00 am

Location of Operation

#### 16 Arumugam Rd #01-02

Alignment Tool (S) Pte Ltd

Equipment Use

1-2 x lorry crane

1 x lowbed (to transport forklift)

1 x 7ton forklift

1 x platform (to lay out for machine pathway)

Rollers/pallet truck

Wire sling

Webbing sling

Obstacles

No obstacle on unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Lorry Loader Operator

c. Signalman

d. Riggerman

e. Forklift Operator

f. Skill Mover

Job Scope

- Deploy lorry loader at designated unloading bay

- Fork down platform and lay out directly in front of tenant door entrance for machine pathway

- To shift out machine from production to loading bay

- Load machine onto platform

- Using forklift or lorry crane to lift up machine

- Load up machine into container

- Secured machine with strapping belt and chocking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175