# METHOD OF STATEMENT

Date: 11th Jan 2018

Description of work

1) To unload/hoist down 1 unit machine from Malaysia lorry, shift and position into welding room at ground floor

2) To shift out 1 unit machine from welding room to loading bay, hoist up and load onto Malaysia lorry

Machine Spec

Move In

1 unit Gantry Jig Welding

Dim: 400 x 124 x 237 cm

Wt: 2, 600 kgs

Move out

1 unit Axis Positioner

Dim: 180 x 180 x 180 cm

Wt: 2, 800 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

15/01/2018   –   Afternoon slot

Location of Operation

#### No 16 Tuas Ave 7

ST Kinetics

Equipment Use

1 x lorry crane

Pallet truck/rollers

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location

- Hoist up and transfer machine from Malaysia lorry onto lorry chassis

- Transport lorry to loading bay

- Hoist down machine and lower down to near ground level

- Insert pallet truck to machine base

- Shift machine into customer premise at ground floor

- Position machine to location at welding room

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175