# METHOD OF STATEMENT

Date: 21st Aug 2018

Description of work

To uncrate 1 unit machine case, hoist down machine from lorry chassis, insert pallet truck, shift and position to the designated location

Machine Spec

1 unit SV-50DT Drilling Tapping

Dim: 1650 x 2500 x 2370 mm

Wt: 2, 850 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

24/08/2018   -    09:30 am

Location of Operation

#### 2 A.M.K Drive

ITE College Central, Blk E

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/Webbing sling

Shackle

Plywood/metal sheet (to protect tiles flooring)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane and lower down machine to near ground level

- Insert pallet truck to machine base

- Shift and position machine to the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

REMARK:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175