# METHOD OF STATEMENT

Date: 26th Nov 2018

Description of work

To hoist down 2 unit machine from lorry chassis, insert pallet truck to machine base, shift and position into customer premise, 1 unit to ground floor and 1 unit to 2nd floor via cargo lift

Machine Spec

2 unit ACC 63 DX Surface Grinder

Dim: 2738 x 1929 x 1808 mm

Wt: 3, 150 kgs each

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

04/12/2018   -   09:30 am

Location of Operation

#### No 23 Loyang Way

GE Aviation Services Operation LLP

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling/shackle

Metal sheet/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated unloading area

- Hoist up machine from lorry chassis and lower down to near ground level

- Swing crane slowly to loading area

- Insert pallet truck to machine base

- Transport 1 unit machine to ground floor and 1 unit machine to 2nd floor via cargo lift

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175