# METHOD OF STATEMENT

Date: 16th Aug 2023

Description of work

To hoist down 2 unit machine from lorry chassis, insert pallet truck to machine bed, shift and position to 3rd floor via cargo lift

Machine Spec

2 unit ACC12.24 Grinder

Dim: 2740 x 2250 x 1850 mm

Wt: 2, 800 kgs each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/09/23

Time of Operation

10:00 am

Location of Operation

#### No 23 Loyang Way

GE Aviation Services Operation LLP

Equipment Use

1 x lorry loader

Pallet truck

Webbing sling

Shackle

Metal sheet/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Setting up of lorry crane at site

- Rigger to insert sling to crane hook and secured onto rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck to under machine bed

- Transport machine to 3rd floor via cargo lift

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175