# METHOD OF STATEMENT

Date: 21st Nov 2017

Description of work

To unload/hoist down 1 unit machine from lorry chassis, shift and position into customer premise at ground floor

Machine Spec

1 unit Troop Wire Cut TP32

Dim: 1530 x 1200 x 1600 mm

Wt: 1, 300 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

24/11/2017   –   11:30am

Location of Operation

#### No 23 Loyang Way

GE Aviation Engine Services (S) Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location

- Hoist up machine from lorry chassis and lower down to near ground level

- Swing crane slowly to loading area

- Insert pallet truck to machine base

- Shift machine into customer premise at ground floor

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175