# METHOD OF STATEMENT

Date: 24th Aug 2017

Description of work

To unload 5 packages of 1 unit machine from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

Micro-Vu Machine / Model: Excel 502 HC

Wooden Case: 53’ x 49’ x 80’ / Wt: 857 kg

Carton Box: 16’ x 14’ x 10’ / Wt: 6 kg

Carton Box: 24’ x 24’ x 12’ / Wt: 8 kg

Carton Box: 21’ x 20’ x 16’ / Wt: 12 kg

Carton Box: 16’ x 10’ x 10’ / Wt: 2 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

29/08/2017   –   10:30am

Location of Operation

#### No 23 Loyang Way

GE Aviation

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location

- Uncrate machine case

- Hoist up machine from lorry chassis and lower down to near ground level

- Swing crane slowly to loading area

- Insert pallet truck to machine base

- Shift machine into customer premise at ground floor

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175