# METHOD OF STATEMENT

Date: 10th Mar 2020

Description of work

To unload 5 cases of 2 unit machine from 1 x 20ft GP and 1 x 40ft HQ container, uncrate, shift and position into customer premise at ground floor

Machine Spec

Citizen CNC Automatic Lathe machine

Case 1 & 2: 267 x 157 x 218 cm / Wt: 2, 180 kg each

Case 3 & 4: 246 x 157 x 73 cm / Wt: 420 kg each

Case 5: 357 x 140 x 160 cm / Wt: 790 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/03/2020 (Tuesday)

Time of Operation

10:00 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

Panasonic Factory Solution Asia Pacific

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

Metal sheet/canvas (for epoxy floor protection)

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location (back to back with container)

- Pull out case from container onto lorry chassis

- Insert wire sling under machine case

- Hoist down case to the ground

- Uncrate case

- After uncrate, hoist and detach machine from base

- Swing crane away from base and lower down machine to near ground level

- Insert pallet truck under machine base

- Transport machine into production area

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175