# METHOD OF STATEMENT

Date: 18th Jan 2022

Description of work

To hoist down 3 cases of 1 unit machine from lorry chassis, uncrate, shift and position into factory premise at ground floor

Machine Spec

Citizen M32-5M5 CNC Turning

Case 1: 341 x 187 x 231 cm / Wt: 4, 300 kg

Case 2: 1, 000 kg

Case 3: 620 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/01/22

Time of Operation

09:45 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

Panasonic Factory Solution Asia Pacific

Equipment Use

2 x 633 lorry loader

Pallet truck/rollers

Wire/webbing sling

Hand jack / crowbar

Woodblock

Metal sheet/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading bay

- Hoist down case to the ground

- Uncrate case and put aside

- Insert sling to crane hook and secure onto hoisting point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert pallet truck/rollers under machine base

- Transport machine into factory premise

- Position machine at the designated location

- Unload machine from pallet truck/rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175