# METHOD OF STATEMENT

Date: 03rd Jan 2020

Description of work

To unload 4 wooden case of 1 unit machine from 1 x 20ft FR and 1 x 20ft GP container, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit Yasda, Model: YCM430 Ver lll machine

Case 1: 313 x 216 x 322 cm   /   Wt: 7, 700 kg

Case 2: 96 x 96 x 158 cm   /   Wt: 240 kg

Case 3: 179 x 102 x 91 cm   /   Wt: 260 kg

Case 4: 197 x 140 x 67 cm   /   Wt: 240 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/01/2020 (Monday)

Time of Operation

10:00 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

Panasonic Factory Solution Asia Pacific

Equipment Use

1 x 633 lorry loader

1 x low loader

Rollers/pallet truck

1 set wire/webbing sling

Shackle

Metal sheet/canvas (for epoxy floor protection)

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location

- Insert wire sling under machine case

- Hoist up machine case to check on balancing

- When well balancing, hoist down machine case from FR container

- Unload case to the ground

- Uncrate case

- After uncrate, hoist machine and load onto lorry chassis

- Transport machine to location

- Hoist down machine from lorry chassis

- Lower down machine slowly to near ground level

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Transport machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175