# METHOD OF STATEMENT

Date: 29th Oct 2019

Description of work

To unload 1 unit machine from 1 x 20ft container, hoist up to 3rd floor through shutter door opening by using lorry crane

Machine Spec

Versa Genius 50 Cleaning Line

Dim: 370 x 140 x 220 cm

Wt: 520 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

01/11/2019

Time of Operation

09:45 am

Location of Operation

#### 70 Tuas South St 5

Tecnisco Advanced Materials Pte Ltd

Equipment Use

1 x 633 PALFINGER lorry loader

1 set wire sling

1 set webbing sling

1 set shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy 1 x PALFINGER lorry loader at designated hoisting area for the lift

- Barricade off work area

- Insert one end of wire sling to machine hoisting point and another end onto crane hook

- Hoist up machine slowly to check on balancing

- When balancing if achieved, hoist up machine slowly to 3rd level shutter door opening

- Lower down machine to near round level

- Insert pallet truck/rollers under machine base

- Shift and position at designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175