# METHOD OF STATEMENT

Date: 13th Sep 2024

Description of work

To shift out machineries from production onto platform, using forklift to load up onto lorry chassis and deliver to new factory

Machine Spec

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

2 day job

12/10/24

13/10/24

Time of Operation

09:30 am

Location of Operation

#### Blk 115A Commonwealth Drive #01-01

Ascot Precision Engineering Pte Ltd

Equipment Use

4-5 x lorry crane

1 x lowbed (to transport forklift)

1 x 10 ton forklift

1 x platform (to lay out for machine pathway)

Rollers

Pallet truck

Obstacles

No obstacle on unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane and forklift operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Deploy lorry loader at designated unloading bay

- Lay out platform directly in front of tenant door entrance for machine pathway

- To shift out machine from production to loading bay

- Load machine onto platform

- Using forklift to fork up machine from platform

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of tenant premise

- Transport and deliver to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175