# METHOD OF STATEMENT

Date: 11th Mar 2020

Description of work

To shift out machine case to loading area, uncrate, hoist up and detach machine from base, insert rollers under machine base, shift and position back into factory

Machine Spec

UG 800 CNC

Dim 1: 548 x 288 x 362 cm

Wt: 12, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/03/2020

Time of Operation

10:00 am

Location of Operation

#### Blk 16B Tuas Ave 1 #02-40/42

#### JTC Space @ Tuas

Equipment Use

1 x 633 lorry loader – PALFINGER PK100002E

Rollers/pallet truck

1 set wire sling

Shackle

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry loader to 2nd floor via ramp up

- Shift out machine case from inside factory to outside loading area

- Uncrate case

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine slowly to near ground level

- Insert rollers under machine base

- Shift and transport back machine into factory

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175