# METHOD OF STATEMENT

Date: 14th Feb 2020

Description of work

To unload 1 unit machine from low-bed using forklift, load machine onto platform, shift and position into customer premise at ground floor

Machine Spec

1 unit TTC10 machine

Dim: 4m x 2.65m x 2.53m

Wt: 9, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/02/2020

Time of Operation

07:00 am

Duration of operation

07:00am   –   09:00am

Location of Operation

#### Blk 5012 A.M.K Ave 5 #01-10

LHY Engineering (S) Pte Ltd

Equipment Use

1 x low-bed

1 x lorry crane

1 x 10 ton forklift

1 x platform (to lay out for machine pathway)

1 set heavy duty rollers

Obstacles

No obstacle on unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- To lay out platform directly in front of tenant door entrance for machine pathway

- Using forklift to fork down machine from low-bed

- Transport machine to unloading area

- Lower down machine onto platform

- Insert rollers under machine base

- Shift out machine from platform into tenant premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175