# METHOD OF STATEMENT

Date: 11th Feb 2019

Description of work

Unload 10 ton forklift from low-bed, using forklift to unload machine from 1 x 20ft FR container, uncrate, shift and position into premise at ground floor

Machine Spec

1 unit Inte J-200 CNC Lathe

Dim: 463 x 271 x 292 cm

Wt: 8, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

13/02/2019   -   09:30 am

Location of Operation

#### Blk 5014 A.M.K Ave 5 #01-01

Small Tool Technologies Pte Ltd

Equipment Use

1 x low-bed

1 x lorry crane

1 x 10 ton forklift

1 set heavy duty rollers

Obstacles

No obstacle on unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Certified forklift operator

# JOB SCOPE

- Low-bed to transport 10 ton forklift into premise

- Unload forklift from low-bed

- Using forklift to fork down machine from FR container

- Uncrate machine case

- Fork up and detach machine from base

- Transport machine to loading bay

- Lower down machine at loading bay

- Insert rollers to machine base

- Shift machine into inside factory

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175