# METHOD OF STATEMENT

Date: 05th Nov 2021

Description of work

To hoist up equipment from ground floor to 2nd & 3rd floor through shutter door opening by using platform and lorry crane

Equipment Spec

Refer to quotation

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/11/21

Time of Operation

09:45 am

Location of Operation

#### 1 Changi North St 2

ETLA Limited

Equipment Use

633 FALFINGER lorry loader

1 x platform

Wire/webbing sling

Shackle

Rollers/pallet ruck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Load equipment onto platform

- Secure equipment onto platform with strapping belt

- Hoist up platform to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck under equipment base

- Shift out from platform

- Position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175