# METHOD OF STATEMENT

Date: 27th Dec 2021

Description of work

To shift out and hoist down 11 unit machine from 2nd floor by using platform and mobile/lorry crane, load onto lorry chassis and deliver to new location, unload 2 unit machine at ground floor and hoist up 9 unit machine to 2nd floor

Machine Spec

New factory ground floor

1 unit Eumach VCM-1600

Dim: 450 x 330 x 300 cm / Wt: 15 ton

1 unit Makino Slim3 5N

Hoist up to new factory 2nd floor

2 unit Eumach HSM800

Dim: 300 x 220 x 250 cm / Wt: 5.5 ton

2 unit Eumach HSM600

Dim: 240 x 200 x 220 cm / Wt: 3.5 ton

1 unit Milling

1 unit Okamoto 450 Grinder

1 unit Okamoto 350 Grinder

1 unit CMM

1 unit D2-5AX

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

31/12/21 – 07/01/22

Time of Operation

09:40 am

Location of Operation

ETLA Ltd

Move out: 2 Changi North St 2

Move in: 2A Changi North St 1

Equipment Use

1 x 50 ton mobile crane

1 x 633 FALFINGER lorry loader

2 x lorry crane

1 x low loader

1 x platform

1 set wire sling/shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 2nd floor building opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Tied back platform to a competent point and secured using 1 x 633 lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- Insert sling to crane hook and secure onto machine hoisting point

- Hoist up machine slowly to check on balancing

- When all are balance well, mobile crane to hoist and lower down machine to the ground

- Load machine onto lorry chassis

- Secured machine with strapping belt and deliver to new location

- Unload and hoist up machine to 2nd floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175