# METHOD OF STATEMENT

Date: 02nd Dec 2022

Description of work

To shift out and hoist down equipment from 3rd floor by using platform and mobile/lorry crane, load onto trailer/lorry chassis and deliver to new factory

Spec

2 unit Storage System

Dim: 420 x 170 x 90 cm

Dim: 350 x 170 x 100 cm

Dim: 320 x 170 x 100 cm

Dim: 230 x 170 x 100 cm

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/12/22

Time of Operation

09:30 am

Location of Operation

Flowserve Pte Ltd

Move out: 33 Changi South Ave 2

Move in: 12 Tuas Loop

Equipment Use

1 x 50 ton mobile crane

2-3 x lorry crane

1 x trailer

1 x platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety helmet

Safety shoe

Safety harness

Self Retracting Lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

All our workers are fully equip with standard P.P.E                                                                                                                          Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 3rd shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out equipment from production and load onto platform

- Insert sling to crane hook and secure onto equipment hoisting point

- Hoist up slowly to check on balancing

- When all are balance well, mobile crane to hoist and lower down equipment to the ground

- Load onto trailer/lorry chassis

- Secured with strapping belt prior to moving out of premise

- Transport and deliver to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175