# METHOD OF STATEMENT

Date: 30th Dec 2021

Description of work

To shift out and hoist down machineries from 3rd floor by using platform and lorry crane, load onto lorry chassis and deliver to new factory

Machine Spec

Refer to quotation

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

11/01/22

Hoist down 6 unit machine

Small machine via cargo lift

22-25/01/22

Hoist down 8 unit machine

Small machine via cargo lift

Time of Operation

09:40 am

Location of Operation

690 West Camp Rd #03-02/03

Coway Engineering & Marketing Pte Ltd

Equipment Use

1 x 633 lorry loader (PALFINGER PK110002SHH)

1 x 633 lorry loader (PALFINGER PK100002F)

3 x lorry crane

1 x platform/

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy 2 x lorry crane at designated hoisting area for the lift

- 1 x lorry crane to hoist up platform to 3rd floor building opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- Insert sling to crane hook and secure onto machine hoisting point

- Hoist up machine slowly to check on balancing

- When all are balance well, to hoist up and lower down machine to ground floor

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of site premise

- Transport and deliver machine to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175