# METHOD OF STATEMENT

Date: 21st Nov 2022

Description of work

1) To hoist down 1 unit machine from 6th floor through wall hacking opening by using platform and mobile/lorry crane

2) To hoist up 1 unit new machine to 6th floor through wall hacking opening by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate

29/11/22  –  17/12/22

Time of Operation

09:30 am

Location of Operation

Blk 30 Kallang Place #06-16

Youprint Production Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45ton lorry loader

1 x lorry

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

All our workers are fully equip with standard P.P.E (safety shoe, helmet, safety harness)                                                                                                                           Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Hoist up platform to 6th floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Mobile crane to hoist up machine to 6th floor

- Lower down machine onto platform

- Insert pallet truck under machine bed

- Shift out machine from platform and position into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175