# METHOD OF STATEMENT

Date: 11th Sep 2023

Description of work

To shift out and hoist down machineries from 3rd floor through wall hacking opening by using platform and mobile/lorry crane, load onto lorry chassis and deliver to Eric Wong warehouse

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/10/23   -   To shift out small machine to loading bay via cargo lift

07/10/23   -   To shift out and hoist down machine using platform and mobile/lorry crane

Time of Operation

09:40 am

Location of Operation

Blk 30 Loyang Way #03-22

Otori Fine Precision Manufacturing

Equipment Use

1 x 50 ton mobile crane

1 x 633 FALFINGER lorry loader

3-4 x lorry crane

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 3rd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine to near platform

- Use of manual pulley to slowly pull and load machine into platform

- Insert sling to crane hook and secure onto machine rigging point

- Hoist up machine slowly to check on balancing

- When all are balance well, mobile crane to hoist and lower down machine to ground floor

- Load machine onto lorry chassis

- Secured machine with strapping belt and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175