# METHOD OF STATEMENT

Date: 15th Feb 2024

Description of work

To shift out and hoist down machineries from 5th floor through swing gate opening by using platform and mobile crane, load up onto Malaysia lorry

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

01/03/24   -   To shift out small machine and accessories to loading bay via cargo lift and using

lorry crane to hoist and load up onto Malaysia lorry

02/03/24   -   To shift out and hoist down machines from 5th floor using platform and mobile

crane and load up onto Malaysia lorry

Time of Operation

10:00 am

Location of Operation

Blk 512 Chai Chee Lane #05-15

Technova Solutions Pte Ltd

Equipment Use

2 x 50ton mobile crane

2 x lorry crane

1 x platform

1 set wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy 2 x mobile crane at designated hoisting area for the lift

- 1 x mobile crane to hoist up platform to 5th floor through swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on building opening slab

- Tied back platform to a competent point and secured using 1 x mobile crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine to near platform

- Use of manual pulley to slowly pull and load machine into platform

- Riggerman will insert sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine slowly to check on balancing

- When all are balance well, mobile crane to hoist and lower down machine to ground floor

- Load up machine onto Malaysia lorry

- Released all sling from rigging point of machine

- Secured machine with strapping belt

- Transport out and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175