# METHOD OF STATEMENT

Date: 23rd June 2023

Description of work

To hoist down 5 unit machine from 3rd floor through wall hacking opening by using platform and mobile/lorry crane, load up onto lorry chassis

Machine Spec

1 unit TP750 CNC

1 unit Okamoto Surface Grinder

1 unit PM860

1 unit Powertools Surface Grinder

1 unit 5HP Milling

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

01/07/23

Time of Operation

09:40 am

Location of Operation

Blk 9003 Tampines St 93 #03-160

Equipment Use

1 x 50ton mobile crane

2-3 x lorry loader

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Hoist up platform to 3rd floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on wall opening slab

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load into platform

- Insert sling to crane hook and secure onto machine rigging point

- Hoist up machine from platform and lower down to the ground

- Load machine onto lorry chassis

- Secured machine with strapping belt and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175