# METHOD OF STATEMENT

Date: 10th Apr 2025

Description of work

To shift out machines and accessories from tenant premise at ground floor to loading area, hoist up and load onto lorry chassis

Machine Spec

1 unit Kent 84 Surface Grinder

5 unit small Surface Grinder

1 unit ARD EDM

1 unit Novick AR1300 EDM

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate 1 day job

09/05/25

Time of Operation

10:00 am

Location of Operation

#### Blk 7 Kallang Place #01-06

Lek Hung Moulding Pte Ltd

Equipment Use

3-4 x 633 lorry loader

1 set wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lift

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Shift out machine from tenant premise at ground floor to loading area

- Rigger to attached sling to lorry crane hook

- Secured sling to rigging point of machine

- Lorry crane to hoist up machine and load onto lorry chassis

- Secured machine with strapping belt prior to being transport out of site premise

- Once job done, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175