# METHOD OF STATEMENT

Date: 16th Jan 2024

Description of work

1) To shift out and hoist down 2 units machines from 3rd floor, shift and position into ground floor premise @ #02-19

2) To shift out 2 unit machine from #02-19, hoist and load up onto lorry chassis, transport and deliver to another location

Machine Spec

To be hoist down from 3rd floor

1 unit Alpha 1550xx CNC Lathe machine

Dim: 360 x 200 x 170 cm / Wt: 4, 500 kg

1 unit CZ6285 Lathe machine

Dim: 400 x 150 x 170 cm / Wt: 5, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/01/24 (Thursday)

Time of Operation

09:30 am

Location of Operation

Blk 1 Kaki Bukit Ave 3 #02-19 & #03-06

Sekai Marine Pte Ltd

Equipment Use

2 x 633 FALFINGER lorry loader

2 x lorry

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lift

- 1 x lorry crane to hoist up platform to 3rd floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Lorry crane to hoist down machine and load lorry chassis

- Transfer machine to #02-19

- Unload machine from lorry chassis

- Shift and position into factory premise

- To shift out 2 unit machine from #02-19 and load up onto lorry

- Transport out and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175