# METHOD OF STATEMENT

Date: 26th Feb 2024

Description of work

To shift out and hoist down 4 unit machine from 2nd floor, using forklift to fork and load up onto Malaysia lorry

Machine Spec

1 unit Makino EDAF3 machine

1 unit Sodick AQ327L machine

1 unit Sodick K1C machine

1 unit Citizen machine

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/03/24 (Wednesday)

Time of Operation

10:00 am

Location of Operation

Blk 10 Kaki Bukit Ave 1 #02-01

Super Union Precision Engineering Pte Ltd

Equipment Use

2 x 633 FALFINGER lorry loader

1 x 7 ton forklift

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lift

- 1 x lorry crane to hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Lorry crane to hoist down machine

- Using forklift to fork up machine from platform onto Malaysia lorry

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Remark

Wo Fook Contractor to dismantle sun breaker and make good upon job completion

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175