# METHOD OF STATEMENT

Date: 28th Jan 2022

Description of work

To hoist down 1 unit machine from 3rd floor through swing gate opening by using platform and mobile/lorry crane

Machine Spec

1 unit Mitsubishi FA20S

Dim: 240 x 180 x 210 cm

Wt: 3, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/02/22

Time of Operation

09:45 am

Location of Operation

#### Blk 20 Woodlands Link #03-35

Sintech Precision Engineering Pte Ltd

Equipment Use

1 x 50ton mobile crane

1 x 633 FALFINGER lorry loader

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 3rd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift machine into platform

- Mobile crane to hoist down machine to the ground

- Load machine onto lorry chassis and secured with strapping belt

- Transport and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175