# METHOD OF STATEMENT

Date: 28th Mar 2023

Description of work

To unload/hoist down 2 units machines from lorry chassis, shift and position into tenant premises at #01-11

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

29/04/23 – 06/05/23

Time of Operation

09:30 am

Location of Operation

Blk 3015A Ubi Rd 1, From #01-11

Unique Precision Pte Ltd

Equipment Use

1 x 633 FALFINGER lorry loader

1 x lorry

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lift

- Hoist down machine from lorry chassis onto laid out platform

- Insert rollers under machine bed

- Shift out machine from platform

- Transport machine into tenant new premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175