# METHOD OF STATEMENT

Date: 05th July 2024

Description of work

1) To shift out and hoist down 9 unit machine from 2nd floor to the ground, load up onto Malaysia lorry

2) To shift out smaller machine from 2nd floor to loading bay via cargo lift, hoist and load up onto Malaysia lorry

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/07/24 (Saturday)

Time of Operation

09:45 am

Location of Operation

Blk 3015A Ubi Rd 1 #02-11

DM Plastic Mould Pte Ltd

Equipment Use

1 x 50ton mobile crane

2 x 633 lorry loader

Malaysia lorry

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- Mobile crane to hoist down machine

- Load up machine onto Malaysia lorry

- Secured machine with strapping belt prior to moving out of premise

- Transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175