# METHOD OF STATEMENT

Date: 31st Dec 2020

Description of work

1) To shift out machineries from ground floor to loading bay onto platform and hoist up onto lorry chassis by using lorry crane

2) To shift out and hoist down machineries from 2nd floor through swing gate opening by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

12-18/01/2021

Time of Operation

Location of Operation

#### Blk 5004 A.M.K Ave 5 #01-10 & #02-06

GS Precision Pte Ltd

Equipment Use

1 x 50 ton mobile crane

2-3 x lorry crane

1 x low loader

1 x platform

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE FOR LEVEL 1

# Deployment of 50 Ton Lorry crane and Low Loader at designated hoisting area

# Setting up of Pathway for Machine exit

# Jacking up Machine with Manual Jack/Crowbar

# Insert rollers underneath Machine

# Push Machine out of #01-10 onto Low-Loader parked at Loading Bay

# 50 Ton Lorry Crane to Hoist Machine from Static Low-Loader onto movable Low-Loader

# Secure Machine onto Low-Loader

# Leave Premises

# JOB SCOPE FOR LEVEL 2

- Deployment of mobile/lorry crane at designated hoisting area

- 1 x mobile crane to hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 1 x 50 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from tenant premise

- Load machine onto platform

- Mobile crane to hoist down machine from 2nd floor

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to being transport and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Remark:

To dismantle sun breaker and make good upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175