# METHOD OF STATEMENT

Date: 13th Dec 2022

Description of work

To hoist down 1 unit machine from 2nd floor through wall hacking opening by using platform and mobile/lorry crane, load onto lorry chassis and deliver to another location

Machine Spec

1 unit Focaseiki TP750

Dim: 170 x 235 x 240 cm

Wt: 4, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/12/22 (Saturday)

Time of Operation

09:40 am

Location of Operation

Blk 9004 Tampines St 93 #02-122

Eson Precision Engineering

Equipment Use

1 x 50 ton mobile crane

1-2 x 633 FALFINGER lorry loader

1 x platform

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 2nd floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on wall opening slab

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load into platform

- Insert sling to crane hook and secure onto machine rigging point

- Hoist up machine from platform and lower down to the ground

- Load machine onto lorry chassis

- Secured machine with strapping belt and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175