# METHOD OF STATEMENT

Date: 04th Mar 2020

Description of work

1) To shift out and hoist down 2 unit machine from 2nd floor by using platform and lorry crane

2) To hoist up 2 unit machine to 2nd floor by using platform and lorry crane

To be hoisted down

2 unit Rongfu S5

Dim: 200 x 130 x 200 cm

Wt: 830 kg each

To be hoisted up

1 unit Milling

Dim: 150 x 130 x 200 cm

Wt: 1, 300 kg

1 unit Pinacho Turning

Dim: 210 x 90 x 170 cm

Wt: 1, 300 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

11/03/2020

Time of Operation

09:45 am

Location of Operation

Collection/Delivery of Rongfu S5

From blk E4, #02-06 to blk E3, #01-06

Collection/Delivery of Milling & Turning

From blk E1-A #01-06 to blk E4, #02-06

Equipment Use

1 x 633 FALFINGER lorry loader

1 x platform

Pallet truck/rollers

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

1) Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

2) Barricade off work area with safety cones, red/white tape and lifting in operation signage

3) All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            4) All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

5) Lifting machine and lifting gear with M.O.M certificate

6) Lifting crews shall be composed to the following but not limited by other trained personnel in respect to the number of crew needed for the operation:

a. Lifting Supervisor

b. Crane Operator

c. Signalman

d. Riggerman

# JOB SCOPE

Block E4 #02-06

- Deployment of lorry crane at designated hoisting area for the lift

- 1 x lorry crane to hoist up platform to 2nd floor area opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine form workshop into platform

- Secured machine onto platform with strapping belt

- Hoist down platform together with machine to the ground

- Release sling from crane hook

- Hoist up machine from platform and load onto lorry chassis

- Transport machine to new location

Block E3 #01-06

- Hoist down machine from lorry chassis to the ground

- Insert pallet truck under machine base

- Shift and transport machine into premise

- Position machine at the designated location

Block E1-A #01-06

- Lift up machine to 3” high using crowbar

- Insert pallet truck under machine bed

- Shift out machine to unloading area

- Attached lifting gears to crane hook and secure to machine to be lifted

- Lorry crane to hoist up machine and load onto lorry chassis

- Secure machine with strapping belt

- Transport machine to block E4

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Hoist up platform together with machine to 2nd floor area opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from platform and transfer into workshop

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175