# METHOD OF STATEMENT

Date: 04th Aug 2020

Description of work

1) To hoist up 1 unit Blower and 1 unit forklift to 2nd floor by using platform and lorry crane, using forklift to fork up blower onto 1ft slab

2) To fork out 1 unit Blower from 1ft slab, hoist down and place one side

Blower Spec

Hoist down (old blower)

Dim: 240 x 210 x 230 cm

Wt: 1, 000 kg

Hoist up (new blower)

Dim: 235 x 186 x 240 cm

Wt: 1, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBC

Time of Operation

TBC

Location of Operation

#### 7 Tuas Ave 10

Equipment Use

1 x 633 FALFINGER lorry loader

1 x platform

1 x forklift

Pallet truck

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

1) Barricade off work area with safety cones, red/white tape and lifting in operation signage

2) All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            3) All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

4) Lifting machine and lifting gear with M.O.M certificate

5) Lifting crews shall be composed to the following but not limited by other trained personnel in respect to the number of crew needed for the operation

a. Lifting Supervisor

b. Crane Operator

c. Signalman

d. Riggerman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Load forklift onto platform and secured with strapping belt

- Lorry loader to hoist up platform to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Drive out forklift from platform

- Using forklift to fork out old blower from 1ft slab

- Load blower onto platform and secured with strapping belt

- Lorry loader to hoist down platform together with blower to the ground

- Hoist out blower from platform and put one side

- Load new blower onto platform and secured with strapping belt

- Hoist up to 2nd floor shutter door opening

- Shift out blower from platform

- Using forklift to fork up blower onto 1ft slab and position

- Once done, to load forklift onto platform and secured with strapping belt

- Hoist down platform and load onto lorry chassis

- To perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175