# METHOD OF STATEMENT

Date: 29th Dec 2022

Description of work

To hoist down 1 unit used machine and hoist up 2 unit new machine to 3rd floor through shutter door opening using platform and mobile/lorry crane

Machine Spec

Hoist down

1 unit Femco HL25 CNC Lathe

Wt: 3, 000 kg

Hoist up

1 unit Vturn-NP20 CNC Lathe

Wt: 4, 650 kg

1 unit Cylinder Grinder

Wt: 3, 940 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/01/23

Time of Operation

09:45 am

Location of Operation

#### JTC Space @ 164 Gul Circle #01-09

Beril Engineering (S) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

2 x 633 lorry loader

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 3rd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- Attached sling to crane hook and secured onto rigging point of machine

- Mobile crane to hoist down machine and load onto lorry chassis

- To hoist up new machine to 3rd floor

- Lower down machine onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform and position at the designated location

- Once job done, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175